Network Library Usage Guide

Version: 3.0.0.1

1	Function Description	3

Main Functions	3

Network Library File Description	3

2	Programming Guide	4

Main process of calling a network library interface:	4

Process of the real-time data streaming module	6

Process of the PTZ control module	7

Process of the parameter configuration module	8

Process of the voice intercom and forwarding module	9

Process of the alarm module	10

3	Data Type Definitions	11

4	Error Code Definitions	12

5	Function Description	12

5.1	SDK Initialization	12

ZK_NET_DEV_Init	12

ZK_NET_DEV_DeInit	12

5.2	User Registration	12

ZK_NET_DEV_Login	12

ZK_NET_DEV_Logout	13

ZK_NET_DEV_SetConnectTimeout	13

ZK_NET_DEV_SetReconnect	14

5.3	Real-time Preview	14

ZK_NET_DEV_StartStream	14

ZK_NET_DEV_StopStream	15

5.4	Real-time Preview Data Callback	16

ZK_NET_DEV_SetEventCallBack	16

ZK_NET_DEV_SetStreamCallBack	17

ZK_NET_DEV_SetDataCallBack	19

5.5	Camera Attribute Configuration	20

ZK_NET_DEV_SetConfig	20

ZK_NET_DEV_GetConfig	31

5.6	PTZ Control	34

ZK_NET_DEV_PTZ_Ctrl_Standard	34

ZK_NET_DEV_PTZ_Ctrl_StandardEx	35

ZK_NET_DEV_PTZ_Ctrl_Preset	35

ZK_NET_DEV_PTZ_Ctrl_Extend	36

ZK_NET_DEV_PTZ_Fully_Trans	37

5.7	Voice Intercom	37

ZK_NET_DEV_StartVoice	37

ZK_NET_DEV_StopVoice	38

ZK_NET_DEV_SendVoiceData	39

5.8	Video Recording	39

ZK_NET_DEV_StartRecord	41

ZK_NET_DEV_StopRecord	41

ZK_NET_DEV_GetRecordState	41

5.9	Snapshot	42

ZK_NET_DEV_SnapJpeg	42

ZK_NET_DEV_SnapJpegEx	42

6	Audio Coding Description	43

Audio Collection Format Configuration	43

Audio Collection Process	44

Audio Play Process	44

Audio Encoding	44

Audio Decoding	46

6	Appendix	48

Appendix I	Folder List	48

Appendix II	Vendor Code and Equipment Type Definitions	48

# Function Description

## Main Functions

A network library provides the following functions:

- Communication with cameras

- Code stream acquisition

- Parameter configuration

- Voice intercom

- Voice control

- Video recording

Network libraries are connected only to networks and separated from play libraries. Callback data is sent to the play library for processing. Network libraries can be used by platform software and centralized management clients.

## Network Library File Description

| Network library files | ZK_NET_DEV_sdk.h | Header file |
| --- | --- | --- |
|  | ZKNetLib.lib | LIB library file |
|  | ZKNetLib.dll | DLL library file |
| Public file | hi_dataType.h | Header file |

# Programming Guide

## Main process of calling a network library interface:

Real-time data streaming module

PTZ module

Parameter configuration module

Voice intercom and forwarding module

Alarm module

Set the connection timeout period.

ZK_NET_DEV_SetConnectTimeout

Set the automatic reconnection time.

ZK_NET_DEV_SetReconnect

Initialize a network library.

ZK_NET_DEV_Init

Register user equipment.

ZK_NET_DEV_Login

Deregister user equipment.

ZK_NET_DEV_Logout

Release the network library.

ZK_NET_DEV_DeInit

A network library is divided into five modules by function. The following steps are mandatory when the functions of each module are implemented: initialize the network library -> register user equipment -> deregister equipment -> release network library resources.

- Initializing the network library (ZK_NET_DEV_Init): The whole network library system is initialized.

- Setting the connection timeout period (ZK_NET_DEV_SetConnectTimeout): This step sets the network connection timeout period in the SDK and is optional. The timeout period can be set by users as required. If it is not set by calling the interface, the default timeout period in the SDK is used.

- Registering user equipment (ZK_NET_DEV_Login): When a user registers successfully, a user ID is returned and used as the unique identifier of other operations.

- Real-time data streaming module: It uses callback functions to obtain the real-time data of cameras after real-time data streaming starts. For details, see the process of the real-time data streaming module.

- Pan Tilt Zoom (PTZ) control module: It controls basic PTZ functions, preset position setting, and transparent transmission. For details, see the process of the PTZ control module.

- Parameter configuration module: It sets and obtains the parameters of front-end cameras, including equipment parameters, network parameters, alarm parameters, exception parameters, and user configuration. For details, see the process of the parameter configuration module.

- Voice intercom and forwarding module: It performs exchange and acquisition of voice data with front-end servers. The audio encoding format is customizable. For details, see the process of the voice intercom and forwarding module.

- Alarm module: It processes alarm signals uploaded by front-end servers. Alarms are classified into motion alarms and input alarms. For details, see the process of the alarm module.

## Process of the real-time data streaming module

Register user equipment.

ZK_NET_DEV_Login

Deregister user equipment.

ZK_NET_DEV_Logout

Release the network library.

ZK_NET_DEV_DeInit

Initialize a network library.

ZK_NET_DEV_Init

Start real-time data streaming.

ZK_NET_DEV_StartStream

Stop real-time data streaming.

ZK_NET_DEV_StopStream

Call back real-time data.

ZK_NET_DEV_SetStreamCallBack

The real-time data streaming module uses callback functions to obtain the real-time data of cameras after real-time data streaming starts. Each data frame is complete and contains a header to distinguish data types. Related interfaces are ZK_NET_DEV_StartStream, ZK_NET_DEV_StopStream, and ZK_NET_DEV_SetStreamCallBack.

## Process of the PTZ control module

Release the network library.

ZK_NET_DEV_DeInit

Initialize a network library.

ZK_NET_DEV_Init

Register user equipment.

ZK_NET_DEV_Login

Deregister user equipment.

ZK_NET_DEV_Logout

PTZ function control

Preset position setting control

Transparent transmission control

The PTZ control module can be used after network library initialization and equipment registration. It controls basic PTZ functions, preset position setting, and transparent transmission.

- PTZ function control: up, down, left, and right directions, stop, focusing, lighting, and windscreen wiping (which are extended functions). Related interfaces are ZK_NET_DEV_PTZ_Ctrl_Standard, ZK_NET_DEV_PTZ_Ctrl_StandardEx, and ZK_NET_DEV_PTZ_Ctrl_Extend.

- Preset position control: A maximum of 256 positions can be preset (the number varies with specific equipment). The related interface is ZK_NET_DEV_PTZ_Ctrl_Preset.

- Transparent transmission control: Serial ports are used for PTZ control. The related interface is ZK_NET_DEV_PTZ_Fully_Trans.

## Process of the parameter configuration module

Obtain parameters.

ZK_NET_DEV_GetConfig

Release the network library.

ZK_NET_DEV_DeInit

Initialize a network library.

ZK_NET_DEV_Init

Register user equipment.

ZK_NET_DEV_Login

Deregister user equipment

ZK_NET_DEV_Logout

Set parameters.

ZK_NET_DEV_SetConfig

The prerequisites for parameter configuration are network library initialization and user registration. The handle returned by the user registration interface is used as the first parameter of interface configuration. It is recommended that you call the parameter acquisition interface (ZK_NET_DEV_GetConfig) to obtain a complete parameter structure before the configuration of a certain type of parameters. Then, modify required parameters and call the parameter configuration interface (ZK_NET_DEV_SetConfig) with the modified parameters as input parameters. You will receive a message when parameters are successfully set.

## Process of the voice intercom and forwarding module

Release the network library.

ZK_NET_DEV_DeInit

Initialize a network library.

ZK_NET_DEV_Init

Register user equipment.

ZK_NET_DEV_Login

Deregister user equipment.

ZK_NET_DEV_Logout

Start talkback.

ZK_NET_DEV_StartVoice

Forward data.

ZK_NET_DEV_SendVoiceData

Stop talkback.

ZK_NET_DEV_StopVoice

In the voice intercom and forwarding process, audio data is sent to a camera and must be encoded to comply with the audio encoding format required by the camera. For details about audio coding, see chapter 6 "Audio Coding Description " and the demo.

## Process of the alarm module

Register user equipment.

ZK_NET_DEV_Login

Deregister user equipment.

ZK_NET_DEV_Logout

Release the network library.

ZK_NET_DEV_DeInit

Initialize a network library.

ZK_NET_DEV_Init

Call back alarm information.

ZK_NET_DEV_SetDataCallBack

Stop real-time data streaming.

ZK_NET_DEV_StopStream

Start real-time data streaming.

ZK_NET_DEV_StartStream

Alarm callback covers motion alarms and input alarms.

- Motion alarms: When motion is detected in an area monitored by a camera, a callback function is used to output the data related to the area.

- Input alarms: Input alarms are generated when camera-related parameters are modified.

For usage details, see the usage of the alarm callback function ZK_NET_DEV_SetDataCallBack.

# Data Type Definitions

typedef unsigned char 		HI_U8;

typedef unsigned char 		HI_UCHAR;

typedef unsigned short 		HI_U16;

typedef unsigned int 		HI_U32;

typedef signed char 		HI_S8;

typedef short 				HI_S16;

typedef int 				HI_S32;

#ifndef _M_IX86

typedef unsigned long long 	HI_U64;

typedef long long 			HI_S64;

#else

typedef __int64 			HI_U64;

typedef __int64 			HI_S64;

#endif

typedef char 				HI_CHAR;

typedef char* 				HI_PCHAR;

typedef float 				HI_FLOAT;

typedef double 			HI_DOUBLE;

typedef void 				HI_VOID;

typedef unsigned long 		HI_SIZE_T;

typedef unsigned long 		HI_LENGTH_T;

typedef enum {

HI_FALSE    = 0,

HI_TRUE     = 1,

} HI_BOOL;

#ifndef NULL

#define NULL 			0L

#endif

#define HI_NULL 			0L

#define HI_NULL_PTR 	0L

#define HI_SUCCESS 		0

#define HI_FAILURE 		(-1)

# Error Code Definitions

#define ZK_NET_DEV_PARAM_ERROR 	0x41001

#define HI _NET_DEV_MEMORY_ERROR 	0x41002

#define HI _NET_DEV_NOT_SUPPORT 	0x41003

#define HI _NET_DEV_PARAM_CHECK_ERROR 0x51001	//Parameter input error

#define ZK_NET_DEV_PARAM_CMD_ERROR	0x51002	//No command

#define ZK_NET_DEV_PARAM_PARSE_ERROR	0x51003	//parse command

#define ZK_NET_DEV_NET_CONNECT_FAIL	0x52001	//connect host failure

#define ZK_NET_DEV_NET_TRANSFER_FAIL 	0x52002	//transfer host failure

#define ZK_NET_DEV_NET_RETURN_ERROR 	0x52003	//host return error

#define ZK_NET_DEV_NET_NOT_SUPPORT	0x53000	//device not support the parameter

# Function Description

## SDK Initialization

### ZK_NET_DEV_Init

This function is used only for SDK initialization before other functions are called.

HI_S32  ZK_NET_DEV_Init(

);

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_DeInit

This function is used only for SDK release.

HI_S32  ZK_NET_DEV_DeInit(

);

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

ZK_NET_DEV_Init and ZK_NET_DEV_DeInit are used for Socket initialization and uninitialization respectively. Each can be used only once in a program.

## User Registration

### ZK_NET_DEV_Login

This function is used for user equipment registration.

HI_S32 ZK_NET_DEV_Login(

HI_U32* 				pu32Handle,

const HI_CHAR* 			psUsername,

const HI_CHAR* 			psPassword,

const HI_CHAR* 			psHost,

HI_U16 					u16Port

);

Parameters

pu32Handle

[OUT] Operation handle

psUsername

[IN] User name

psPassword

[IN] Password

psHost

[IN] Host, which can be represented by an IP address or a domain name

u16Port

[IN] Port number

Return Values

- HI_SUCCESS indicates a successful operation.

- ZK_NET_DEV_NET_CONNECT_FAIL indicates a connection failure.

- ZK_NET_DEV_NET_TRANSFER_FAIL indicates a domain name parsing failure.

- ZK_NET_DEV_NET_RETURN_ERROR indicates a host error.

### ZK_NET_DEV_Logout

This function is used for user logout.

HI_S32  ZK_NET_DEV_Logout(

HI_U32 		u32Handle

);

Parameters

u32Handle

[IN] Operation handle

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_SetConnectTimeout

This function is used to set the connection timeout period, in the unit of milliseconds. The default value is 5s.

HI_S32  ZK_NET_DEV_SetConnectTimeout (

HI_U32 		u32Handle

HI_U32 		u32Timeout

);

Parameters

u32Handle

[IN] Operation handle

u32Timeout

[IN] Timeout period, in the unit of milliseconds

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_SetReconnect

This function is used to set the automatic reconnection interval, in the unit of milliseconds. The default value is 10s. The value 0 indicates that reconnection is not performed.

HI_S32  ZK_NET_DEV_SetReconnect (

HI_U32 		u32Handle

HI_U32 		u32Interval

);

Parameters

u32Handle

[IN] Operation handle

u32Interval

[IN] Automatic reconnection interval, in the unit of milliseconds

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

## Real-time Preview

### ZK_NET_DEV_StartStream

This function is used to start real-time data streaming.

HI_S32  ZK_NET_DEV_StartStream (

HI_U32 					u32Handle

HI_S_STREAM_INFO* 	pstruStreamInfo

);

Parameters

u32Handle

[IN] Operation handle

pstruStreamInfo

[IN] Operation parameter

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

//Start streaming.

typedef struct

{

HI_U32 		u32Channel;     		//Channel number, which should be consistent with the number of the queried channel

HI_BOOL 	blFlag;       		//TRUE indicates primary code streams and FALSE indicates secondary code streams.

HI_U32 		u32Mode;       		//Network connection mode

HI_U8 		u8Type;          	//Streaming data type, which can be audio, video, or others

} HI_S_STREAM_INFO;

//Equipment channel number. Currently, only one channel is supported.

#define ZK_NET_DEV_CHANNEL_1    1

//#define ZK_NET_DEV_CHANNEL_2    2

//#define ZK_NET_DEV_CHANNEL_3    3

//#define ZK_NET_DEV_CHANNEL_4    4

//Network connection mode. Currently, only TCP is supported.

#define ZK_NET_DEV_STREAM_MODE_TCP  0

//Streaming data type. Currently, heartbeat data is not supported.

//Secondary code streams do not support alarm data or heartbeat data.

#define ZK_NET_DEV_STREAM_VIDEO_ONLY   	0x01

#define ZK_NET_DEV_STREAM_AUDIO_ONLY    	0x02

#define ZK_NET_DEV_STREAM_VIDEO_AUDIO 	 	0x03

#define ZK_NET_DEV_STREAM_VIDEO_DATA   	0x05

#define ZK_NET_DEV_STREAM_AUDIO_DATA   	0x06

#define ZK_NET_DEV_STREAM_ALL              	0x07

### ZK_NET_DEV_StopStream

This function is used to stop data streaming.

HI_S32  ZK_NET_DEV_StopStream(

HI_U32 		u32Handle

);

Parameters

u32Handle

[IN] Operation handle

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

## Real-time Preview Data Callback

### ZK_NET_DEV_SetEventCallBack

This function is used for event data callback.

HI_S32  ZK_NET_DEV_SetEventCallBack(

HI_U32 				u32Handle

HI_ON_EVENT_CALLBACK cbEventCallBack,

HI_VOID* 			pUserData

);

Parameters

u32Handle

[IN] Operation handle

cbEventCallBack

[IN] Event data callback function

pUserData

[IN] User data

Callback Function

typedef  HI_S32  (*HI_ON_EVENT_CALLBACK) (

HI_U32 		u32Handle,

HI_U32 		u32Event,

HI_VOID* 	pUserData

);

Callback Function Parameters

u32Handle

Operation handle

u32Event

Event

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_CONNECTING | 0 | Connecting |
| ZK_NET_DEV_CONNECTED | 1 | Connected |
| ZK_NET_DEV_CONNECT_FAILED | 2 | Connection failed |
| ZK_NET_DEV_ABORTIBE_DISCONNECTED | 3 | Disconnected |
| ZK_NET_DEV_NORMAL_DISCONNECTED | 4 | Disconnected |
| ZK_NET_DEV_RECONNECTING | 5 | Reconnecting |
| ZK_NET_DEV_RECORD_START | 6 | Recording started |
| ZK_NET_DEV_RECORD_STOP | 7 | Recording stopped |

pUserData

User data

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_SetStreamCallBack

This function is used for code stream data callback.

HI_S32  ZK_NET_DEV_SetStreamCallBack (

HI_U32 				u32Handle

HI_ON_STREAM_CALLBACK cbStreamCallBack,

HI_VOID* 			pUserData

);

Parameters

u32Handle

[IN] Operation handle

cbStreamCallBack

[IN] Code stream data callback function

pUserData

[IN] User data

Callback Function

typedef  HI_S32  (*HI_ON_STREAM_CALLBACK)(

HI_U32 		u32Handle,

HI_U32 		u32DataType,

HI_U8*  	pu8Buffer,

HI_U32 		u32Length,

HI_VOID* 	pUserData

);

Callback Function Parameters

u32Handle

Operation handle

u32DataType

Data type, which can be audio, video, or header file

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_AV_DATA | 0 | Audio and video data |
| ZK_NET_DEV_SYS_DATA | 1 | File data |

pu8Buffer

Data containing frame headers

u32Length

Data length

pUserData

User data

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

- The first packet for a successful connection is of the ZK_NET_DEV_SYS_DATA type.

- The pu8Buffer structure is HI_S_SysHeader if the value of u32DataType is ZK_NET_DEV_SYS_DATA.

typedef struct

{

HI_U32 	u32Width;  		//Video width

HI_U32 	u32Height; 		//Video height

} HI_S_VideoHeader;

typedef struct

{

HI_U32 	u32Format; 		//Audio format

} HI_S_AudioHeader;

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_AUDIO_TYPE_G711 | 0 | G.711 |
| ZK_NET_DEV_AUDIO_TYPE_G726 | 1 | G.726 |
| ZK_NET_DEV_AUDIO_TYPE_AMR | 2 | AMR |

typedef struct

{

HI_U32 			u32SysFlag;

HI_S_VideoHeader 	struVHeader;

HI_S_AudioHeader 	struAHeader;

} HI_S_SysHeader;

u32SysFlag is the macro definition #define  ZK_NET_DEV_SYS_FLAG  0x53565848.

  - The pu8Buffer frame header structure is HI_S_SysHeader if the value of u32DataType is ZK_NET_DEV_AV_DATA.

typedef struct

{

HI_U32 	u32AVFrameFlag;        	//Frame flag

HI_U32 	u32AVFrameLen;          //Frame length

HI_U32 	u32AVFramePTS;       	//Timestamp

HI_U32 	u32VFrameType;       	//Video type, which can be I frames or P frames

} HI_S_AVFrame;

u32AVFrameFlag formats:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_VIDEO_FRAME_FLAG | 0x46565848 | Video data |
| ZK_NET_DEV_AUDIO_FRAME_FLAG | 0x46415848 | Audio data |

u32VFrameType formats:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_VIDEO_FRAME_I | 1 | I frames |
| ZK_NET_DEV_VIDEO_FRAME_P | 2 | P frames |

### ZK_NET_DEV_SetDataCallBack

This function is used for information data callback.

HI_S32  ZK_NET_DEV_SetDataCallBack (

HI_U32 				u32Handle

HI_ON_DATA_CALLBACK cbDataCallBack,

HI_VOID* 			pUserData

);

Parameters

u32Handle

[IN] Operation handle

cbDataCallBack

[IN] Information data callback function

pUserData

[IN] User data

Callback Function

typedef  HI_S32  (*HI_ON_DATA_CALLBACK)(

HI_U32 		u32Handle,

HI_U32 		u32DataType,

HI_U8*  	pu8Buffer,

HI_U32 		u32Length,

HI_VOID* 	pUserData

);

Callback Function Parameters

u32Handle

Operation handle

u32DataType

Data type

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_MOTION_DETECTION | 0 | Motion detection alarm |
| ZK_NET_DEV_INPUT_ALARM | 1 | Input alarm |
| ZK_NET_DEV_KEEP_ALIVE | 2 | Heartbeat packet |

pu8Buffer

Data. If the value of u32DataType is ZK_NET_DEV_MOTION_DETECTION, pu8Buffer is stored as HI_S_ALARM_MD.

typedef struct

{

HI_U32 		u32Area;			//Area

HI_U32 		u32X;			//x coordinate

HI_U32 		u32Y;			//y coordinate

HI_U32 		u32Width;		//Rectangular width

HI_U32 		u32Height;		//Rectangular height

} HI_S_ALARM_MD;

The maximum value of u32Area is 4. Related data is as follows:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_MOTION_AREA_1 | 1 | Area 1 |
| ZK_NET_DEV_MOTION_AREA_2 | 2 | Area 2 |
| ZK_NET_DEV_MOTION_AREA_3 | 3 | Area 3 |
| ZK_NET_DEV_MOTION_AREA_4 | 4 | Area 4 |

u32Length

Data length. If the value of u32DataType is ZK_NET_DEV_MOTION_DETECTION and alarms are generated in two areas, the value of u32Length is:

u32Length = 2*sizeof(HI_S_ALARM_MD)

u32DataType

User data

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

## Camera Attribute Configuration

You can determine whether a camera supports an attribute based on sProduct of ZK_NET_DEV_GET_PRODUCT_VENDOR. For details, see Appendix II "Vendor Code and Equipment Type Definitions."

### ZK_NET_DEV_SetConfig

This function is used to set camera-related parameters.

HI_S32  ZK_NET_DEV_SetConfig (

HI_U32 			u32Handle

HI_U32 			u32Command,

HI_VOID* 		pBuf,

HI_U32 			u32BufLen

);

Parameters

u32Handle

[IN] Operation handle

u32Command

[IN] Operation parameter command

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_CMD_DISPLAY | 0x1001 | Image-related parameters |
| ZK_NET_DEV_CMD_DISPLAY_EXT | 0x1002 | Flip |
| ZK_NET_DEV_CMD_INFRARED | 0x1003 | Infrared |
| ZK_NET_DEV_CMD_VIDEO_PARAM | 0x1004 | Video-related parameters |
| ZK_NET_DEV_CMD_OSD_PARAM | 0x1005 | OSD parameters |
| ZK_NET_DEV_CMD_AUDIO_PARAM | 0x1006 | Audio-related parameters |
| ZK_NET_DEV_CMD_AUDIO_INPUT | 0x1007 | Audio input |
| ZK_NET_DEV_CMD_RESOLUTION | 0x1008 | Image resolution |
| ZK_NET_DEV_CMD_FREQUENCY | 0x1009 | Frequency |
| ZK_NET_DEV_CMD_PTZ_PARAM | 0x1010 | PTZ information |
| ZK_NET_DEV_CMD_MD_PARAM | 0x1011 | Motion alarm information |
| ZK_NET_DEV_CMD_NET_INFO | 0x1012 | Network information |
| ZK_NET_DEV_CMD_HTTP_PORT | 0x1013 | Web page port number |
| ZK_NET_DEV_CMD_SERVER_TIME | 0x1017 | Camera time configuration |
| ZK_NET_DEV_CMD_REBOOT | 0x1018 | Restart |
| ZK_NET_DEV_CMD_RESET | 0x1019 | Restoration to factory defaults |
| ZK_NET_DEV_CMD_NET_EXT | 0x1022 | Network parameter configuration |
| ZK_NET_DEV_CMD_ATTR_EXT | 0x1026 | Input alarm control |

pBuf

[IN] Data to be configured

u32BufLen

[IN] Data length

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

- ZK_NET_DEV_CMD_DISPLAY

typedef struct HI_Display

{

HI_U32 		u32Brightness;	//Lightness, with the value range of [0–255]

HI_U32 		u32Saturation;		//Saturation, with the value range of [0–255]

HI_U32 		u32Contrast;		//Contrast, with the value range of [0–255]. For high definition, the value range is [1–7].

HI_U32 		u32Hue;			//Chromaticity, with the value range of [0–255], unavailable for high definition

} HI_S_Display;

Note: When the value of u32Brightness is –1, the default lightness is used. For details about color support, see the S field described in Appendix II "Vendor Code and Equipment Type Definitions."

Example:

HI_S_Display sDisplay;

// sDisplay.u32Brightness = -1; //The default lightness is used.

sDisplay.u32Brightness = 100;

sDisplay.u32Saturation = 100;

sDisplay.u32Contrast = 100;

sDisplay.u32Hue = 100;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_DISPLAY,

&sDisplay,

sizeof(HI_S_Display));

- ZK_NET_DEV_CMD_DISPLAY_EXT

typedef struct HI_Display_Ext

{

HI_BOOL 	blFlip;			//Vertical flip

HI_BOOL 	blMirror; 		//Horizontal flip

HI_S32 		s32Scene; 		//Scene, which can be automatic, indoor, or outdoor

} HI_S_Display_Ext;

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_SCENE_AUTO | 0 | Automatic |
| ZK_NET_DEV_SCENE_INDOOR | 1 | Indoor |
| ZK_NET_DEV_SCENE_OUTDOOR | 2 | Outdoor |

Example:

HI_S_Display_Ext sDisplayEx;

sDisplayEx.blFlip = HI_FALSE;

sDisplayEx.blMirror = HI_FALSE;

sDisplayEx.s32Scene = ZK_NET_DEV_SCENE_AUTO;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_DISPLAY_EXT,

&sDisplayEx,

sizeof(HI_S_Display_Ext));

Note: For details about equipment support, see the S field described in Appendix II "Vendor Code and Equipment Type Definitions."

- ZK_NET_DEV_CMD_INFRARED

typedef struct HI_Infrared

{

HI_S32 		s32Infrared;		//Infrared state switch

} HI_S_Infrared;

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_INFRARED_AUTO | 0 | Automatic |
| ZK_NET_DEV_INFRARED_ON | 1 | The infrared function is enabled. |
| ZK_NET_DEV_INFRARED_OFF | 2 | The infrared function is disabled. |

Example:

HI_S_Infrared sInfrared;

sInfrared.s32Infrared = ZK_NET_DEV_INFRARED_AUTO;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_INFRARED,

&sInfrared,

sizeof(HI_S_Infrared));

Note: For details about equipment support, see the S field described in Appendix II "Vendor Code and Equipment Type Definitions."

- ZK_NET_DEV_CMD_VIDEO_PARAM

typedef struct HI_Video

{

HI_U32 		u32Channel;		//Channel

HI_BOOL 	blFlag;			//Primary or secondary code stream flag. 0 indicates secondary code streams and 1 indicates primary code streams.

HI_U32 		u32Bitrate;		//Bit rate, in the unit of kbit/s

HI_U32 		u32Frame;		//Frame rate

HI_U32 		u32Iframe;		//Main frame interval (1–300)

HI_BOOL 	blCbr;			//Video encoding control. 0 indicates a variable bit rate and 1 indicates a fixed bit rate.

HI_U32 		u32ImgQuality;	//Video encoding quality (1–6)

} HI_S_Video;

Note: u32Channel is consistent with that in the HI_S_STREAM_INFO parameter of ZK_NET_DEV_StartStream. Their acquisition and configuration are the same.

Example:

HI_S_Video sVideo;

//Note: u32Channel is consistent with HI_S_STREAM_INFO.

sVideo.u32Channel = ZK_NET_DEV_CHANNEL_1;

sVideo.blFlag = HI_TRUE;

sVideo.u32Bitrate = 1024;

sVideo.u32Frame = 25;

sVideo.u32Iframe = 50;

sVideo.blCbr = HI_FALSE;

sVideo.u32ImgQuality = 1;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_VIDEO_PARAM,

&sVideo,

sizeof(HI_S_Video));

- ZK_NET_DEV_CMD_OSD_PARAM

typedef struct HI_OSD

{

HI_BOOL 	blEnTime;		//Overlay time

HI_BOOL 	blEnName;		//Overlay name

HI_CHAR 	sName[64];		//OSD name, which contains a maximum of 18 bytes

} HI_S_OSD;

Example:

HI_S_OSD sOSD;

sOSD.blEnTime = HI_TRUE;

sOSD.blEnName = HI_TRUE;

strcpy(sOSD. sName, “IPCAM”);

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_OSD_PARAM,

&sOSD,

sizeof(HI_S_OSD));

- ZK_NET_DEV_CMD_AUDIO_PARAM

typedef struct HI_Audio

{

HI_U32 		u32Channel;		//Channel

HI_BOOL 	blFlag;			//Primary or secondary code stream flag. 0 indicates secondary code streams and 1 indicates primary code streams.

HI_BOOL 	blEnable;			//Whether to collect audio signals

HI_U32 		u32Type;			//Audio format

} HI_S_Audio;

Note: u32Channel is consistent with that in the HI_S_STREAM_INFO parameter of ZK_NET_DEV_StartStream. Their acquisition and configuration are the same.

u32Type formats:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_AUDIO_TYPE_G711 | 0 | G.711 |
| ZK_NET_DEV_AUDIO_TYPE_G726 | 1 | G.726 |
| ZK_NET_DEV_AUDIO_TYPE_AMR | 2 | AMR |

Example:

HI_S_Audio sAudio;

//Note: u32Channel is consistent with HI_S_STREAM_INFO.

sAudio.u32Channel = ZK_NET_DEV_CHANNEL_1;

sAudio.blFlag = HI_TRUE;

sAudio.blEnable = HI_TRUE;

sAudio.u32Type = ZK_NET_DEV_AUDIO_TYPE_G711;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_AUDIO_PARAM,

&sAudio,

sizeof(HI_S_Audio));

- ZK_NET_DEV_CMD_AUDIO_INPUT

typedef enum HI_AudioInput

{

AUDIO_INPUT_MIC = 100,		//Microphone input

AUDIO_INPUT_LINE = 10		//Linear input

} HI_E_AudioInput;

Example:

HI_S32 audioInput = AUDIO_INPUT_MIC;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_AUDIO_INPUT,

&audioInput,

sizeof(HI_S32));

- ZK_NET_DEV_CMD_RESOLUTION

typedef struct HI_Resolution

{

HI_U32 		u32Channel;		//Channel

HI_BOOL 	blFlag;			//Primary or secondary code stream flag. 0 indicates secondary code streams and 1 indicates primary code streams.

HI_U32 		u32Resolution;	//Resolution

} HI_S_Resolution;

Note: u32Channel is consistent with that in the HI_S_STREAM_INFO parameter of ZK_NET_DEV_StartStream. Their acquisition and configuration are the same.

Values of u32Resolution:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_RESOLUTION_VGA | 0 | VGA: 640x480 |
| ZK_NET_DEV_RESOLUTION_QVGA | 1 | QVGA: 320x240 |
| ZK_NET_DEV_RESOLUTION_QQVGA | 2 | QQVGA: 160x120 and 160x112 |
| ZK_NET_DEV_RESOLUTION_D1 | 3 | D1: 704x576 and 704x480 |
| ZK_NET_DEV_RESOLUTION_CIF | 4 | CIF: 352x288 and 352x240 |
| ZK_NET_DEV_RESOLUTION_QCIF | 5 | QCIF: 176x144, 176x120, and 176x112 |
| ZK_NET_DEV_RESOLUTION_720P | 6 | 720P: 1280x720 |

Example:

HI_S_Resolution sResolution;

//Note: u32Channel is consistent with HI_S_STREAM_INFO.

sResolution.u32Channel = ZK_NET_DEV_CHANNEL_1;

sResolution.blFlag = HI_TRUE;

sResolution.u32Resolution = ZK_NET_DEV_RESOLUTION_CIF;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_RESOLUTION,

&sResolution,

sizeof(HI_S_Resolution));

Note: For details about the equipment-supported resolution, see the S field described in Appendix II "Vendor Code and Equipment Type Definitions."

- ZK_NET_DEV_CMD_FREQUENCY

typedef enum HI_Frequency

{

FREQ_50HZ_PAL = 50,			//50HZ

FREQ_60HZ_NTSC = 60		//60HZ

} HI_E_Frequency;

Example:

HI_U32 sFrequency = FREQ_50HZ_PAL;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_FREQUENCY,

&sFrequency,

sizeof(HI_U32));

Note: Currently, the equipment corresponding to the S1 and S2 fields do not support frequency configuration. For details, see Appendix II "Vendor Code and Equipment Type Definitions."

- ZK_NET_DEV_CMD_PTZ_PARAM

typedef struct HI_PTZ

{

HI_U32 		u32Protocol;		//Protocol

HI_U32 		u32Address;		//Address code, with the value range of [0–255]

HI_U32 		u32Baud;			//Baud rate

HI_U32 		u32DataBit;		//Data bit

HI_U32 		u32StopBit;		//Stop bit

HI_U32 		u32Parity;		//Parity check

} HI_S_PTZ;

Values of u32Protocol:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_PTZ_PRO_PELCOD | 0 | PELCO-D |
| ZK_NET_DEV_PTZ_PRO_PELCOP | 1 | PELCO-P |

Data of u32Baud:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_PTZ_B110 | 110 | 110 |
| ZK_NET_DEV_PTZ_B300 | 300 | 300 |
| ZK_NET_DEV_PTZ_B1200 | 1200 | 1200 |
| ZK_NET_DEV_PTZ_B2400 | 2400 | 2400 |
| ZK_NET_DEV_PTZ_B4800 | 4800 | 4800 |
| ZK_NET_DEV_PTZ_B9600 | 9600 | 9600 |
| ZK_NET_DEV_PTZ_B19200 | 19200 | 19200 |
| ZK_NET_DEV_PTZ_B38400 | 38400 | 38400 |
| ZK_NET_DEV_PTZ_B57600 | 57600 | 57600 |

Data of u32DataBit:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_PTZ_DATA_5 | 5 |  |
| ZK_NET_DEV_PTZ_DATA_6 | 6 |  |
| ZK_NET_DEV_PTZ_DATA_7 | 7 |  |
| ZK_NET_DEV_PTZ_DATA_8 | 8 |  |

Data of u32StopBit:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_PTZ_STOP_1 | 1 |  |
| ZK_NET_DEV_PTZ_STOP_2 | 2 |  |

Data of u32Parity:

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_PTZ_PARITY_NONE | 0 | No parity check |
| ZK_NET_DEV_PTZ_PARITY_ODD | 1 | Odd parity check |
| ZK_NET_DEV_PTZ_PARITY_EVEN | 2 | Even parity check |

Example:

HI_S_PTZ sPtz;

sPtz. u32Protocol = ZK_NET_DEV_PTZ_PRO_PELCOD;

sPtz. u32Address = 1;

sPtz. u32Baud = ZK_NET_DEV_PTZ_B9600;

sPtz. u32DataBit = ZK_NET_DEV_PTZ_DATA_8;

sPtz. u32StopBit = ZK_NET_DEV_PTZ_STOP_1;

sPtz. u32Parity = ZK_NET_DEV_PTZ_PARITY_NONE;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_PTZ_PARAM,

&sPtz,

sizeof(HI_S_PTZ));

- ZK_NET_DEV_CMD_MD_PARAM

typedef struct HI_MD_PARAM

{

HI_U32 		u32Channel;		//Channel

HI_U32 		u32Area;			//Rectangular area (1–4)

HI_BOOL 	blEnable;			//0 indicates disabling motion detection and 1 indicates enabling motion detection.

HI_U32 		u32Sensitivity; 	//Sensitivity (0–100)

HI_U32 		u32X;			//x coordinate

HI_U32 		u32Y;			//y coordinate

HI_U32 		u32Width;		//Rectangular width

HI_U32 		u32Height;		//Rectangular height

} HI_S_MD_PARAM;

Example:

HI_S_MD_PARAM sMdParam;

//Note: u32Channel is consistent with HI_S_STREAM_INFO.

sMdParam.u32Channel = ZK_NET_DEV_CHANNEL_1;

sMdParam.u32Area = 1;

sMdParam.bEnable = HI_TRUE;

sMdParam.u32Sensitivity = 50;

sMdParam.u32X = 100;

sMdParam.u32Y = 100;

sMdParam.u32Width = 200;

sMdParam.u32Height = 200;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_MD_PARAM,

&sMdParam,

sizeof(HI_S_MD_PARAM));

Note: Secondary code streams do not support motion detection.

- ZK_NET_DEV_CMD_NET_INFO

typedef struct tagHI_NETINFO

{

HI_CHAR 	aszServerIP[40];		//IP address

HI_CHAR 	aszNetMask[40];		//Subnet mask

HI_CHAR 	aszGateWay[40];		//Gateway

HI_CHAR 	aszMacAddr[40];		//MAC address

HI_CHAR 	aszFDNSIP[40];		//first DNSIP

HI_CHAR 	aszSDNSIP[40];		//DNSIP

HI_S32  	s32DhcpFlag;			//DHCP

HI_S32  	s32DnsDynFlag;		//DNS dynamic allocation flag*/

}HI_S_NETINFO, *PHI_S_NETINFO;

Example:

HI_S_NETINFO sNetInfo;

strcpy(sNetInfo. aszServerIP, “192.168.1.88”);

… …

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_NET_INFO,

&sNetInfo,

sizeof(HI_S_NETINFO));

- ZK_NET_DEV_CMD_HTTP_PORT

typedef struct HI_HTTPPORT

{

HI_U32 		u32HttpPort;

} HI_S_HTTPPORT;

Example:

HI_S_HTTPPORT sHttpPort;

sHttpPort.u32HttpPort = 80;

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_HTTP_PORT,

&sHttpPort,

sizeof(HI_S_HTTPPORT));

- ZK_NET_DEV_CMD_SERVER_TIME

It is used to configure the camera time.

typedef struct hiSERVERTIME_INFO_S

{

HI_CHAR sTime[32];			//Camera time, sample format: 2011.03.11.09.12.08

} HI_S_SERVERTIME;

sTime indicates the camera time. Sample format: 2011.03.11.09.12.08, that is 2011-3-11 09:12:08.

Example:

HI_S_SERVERTIME sServerTime;

memcpy(sServerTime.sTime, “2011.03.11.09.12.08” , sizeof(sServerTime.sTimezone));

ZK_NET_DEV_SetConfig (	lHandle,

ZK_NET_DEV_CMD_SERVER_TIME,

&sServerTime,

sizeof(HI_S_SERVERTIME));

- ZK_NET_DEV_CMD_REBOOT

It is used for camera restart.

Example:

ZK_NET_DEV_SetConfig (lHandle, ZK_NET_DEV_CMD_REBOOT,NULL,0);

- ZK_NET_DEV_CMD_RESET

It is used to restore to factory defaults.

Example:

ZK_NET_DEV_SetConfig (lHandle, ZK_NET_DEV_CMD_RESET,NULL,0);

- ZK_NET_DEV_CMD_NET_EXT

typedef struct HI_NET_EXT

{

HI_S_NETINFO  sNetInfo;

HI_S_HTTPPORT sHttpPort;

}HI_S_NET_EXT;

typedef struct HI_HTTPPORT

{

HI_U32 		u32HttpPort;

} HI_S_HTTPPORT;

typedef struct tagHI_NETINFO

{

HI_CHAR 	aszServerIP[40];		//IP address

HI_CHAR 	aszNetMask[40];		//Subnet mask

HI_CHAR 	aszGateWay[40];		//Gateway

HI_CHAR 	aszMacAddr[40];		//MAC address

HI_CHAR 	aszFDNSIP[40];		//first DNSIP

HI_CHAR 	aszSDNSIP[40];		//DNSIP

HI_S32  	s32DhcpFlag;			//DHCP

HI_S32  	s32DnsDynFlag;		//DNS dynamic allocation flag*/

}HI_S_NETINFO, *PHI_S_NETINFO;

Example:

HI_S_NET_EXT sNetExt;

strcpy(sNetExt.sNetInfo. aszServerIP, “192.168.1.88”);

… …

ZK_NET_DEV_SetConfig (	lHandle,  			// ZK_NET_DEV_GetConfig

ZK_NET_DEV_CMD_NET_EXT,

& sNetExt,

sizeof(HI_S_NET_EXT));

- ZK_NET_DEV_CMD_ATTR_EXT

It is used to configure the input alarm switch.

typedef struct HI_ATTR_EXT

{

HI_U32  u32Enable;	//1 indicates that the input alarm switch is enabled and 0 indicates that it is disabled.

HI_U32  u32Flag;		//0 indicates that the input alarm switch is stopped and 1 indicates that it is started.

}HI_S_ATTR_EXT;

Example:

HI_S_ATTR_EXT sAttrExt;

sAttrExt.u32Enable = 1;

sAttrExt.u32Flag = 0;

ZK_NET_DEV_SetConfig (	lHandle,

ZK_NET_DEV_CMD_ATTR_EXT,

& sAttrExt,

sizeof(HI_S_ATTR_EXT));

### ZK_NET_DEV_GetConfig

This function is used to obtain camera-related parameters.

HI_S32  ZK_NET_DEV_GetConfig (

HI_U32 			u32Handle

HI_U32 			u32Command,

HI_VOID* 		pBuf,

HI_U32 			u32BufLen

);

Parameters

u32Handle

[IN] Operation handle

u32Command

[IN] Operation parameter command

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_GET_PRODUCT_VENDOR | 0x1000 | Vendor information |
| ZK_NET_DEV_CMD_DISPLAY | 0x1001 | Image-related parameters |
| ZK_NET_DEV_CMD_DISPLAY_EXT | 0x1002 | Flip |
| ZK_NET_DEV_CMD_INFRARED | 0x1003 | Infrared |
| ZK_NET_DEV_CMD_VIDEO_PARAM | 0x1004 | Video-related parameters |
| ZK_NET_DEV_CMD_OSD_PARAM | 0x1005 | OSD parameters |
| ZK_NET_DEV_CMD_AUDIO_PARAM | 0x1006 | Audio-related parameters |
| ZK_NET_DEV_CMD_AUDIO_INPUT | 0x1007 | Audio input |
| ZK_NET_DEV_CMD_RESOLUTION | 0x1008 | Image resolution |
| ZK_NET_DEV_CMD_FREQUENCY | 0x1009 | Frequency |
| ZK_NET_DEV_CMD_PTZ_PARAM | 0x1010 | PTZ information |
| ZK_NET_DEV_CMD_MD_PARAM | 0x1011 | Motion alarm information |
| ZK_NET_DEV_CMD_NET_INFO | 0x1012 | Network configuration information |
| ZK_NET_DEV_CMD_HTTP_PORT | 0x1013 | Web page port number |
| ZK_NET_DEV_CMD_DEVICE_INFO | 0x1014 | Equipment information |
| ZK_NET_DEV_CMD_PRODUCTID | 0x1015 | Product ID |
| ZK_NET_DEV_CMD_USERNUM | 0x1016 | Number of user connections |
| ZK_NET_DEV_CMD_SERVER_TIME | 0x1017 | Camera time acquisition |
| ZK_NET_DEV_CMD_NET_EXT | 0x1022 | Network parameter acquisition |
| ZK_NET_DEV_CMD_ATTR_EXT | 0x1026 | Input alarm parameter acquisition |

pBuf

[OUT] Data to be obtained

u32BufLen

[IN] Data length

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

The structure used to obtain related information is the same as that used in configuration. For details, see the definitions of the related structures of ZK_NET_DEV_SetConfig. The following structures are not used in configuration:

- ZK_NET_DEV_GET_PRODUCT_VENDOR

typedef struct HI_ProductVendor

{

HI_CHAR 	sProduct[32];		//Product ID

HI_CHAR	sVendor[32];		//Vendor ID

}HI_S_ProductVendor;

Example:

HI_S_ProductVendor sProduct;

ZK_NET_DEV_GetConfig (	lHandle,

ZK_NET_DEV_GET_PRODUCT_VENDOR,

&sProduct,

sizeof(HI_S_ProductVendor));

- ZK_NET_DEV_CMD_DEVICE_INFO

typedef struct tagHI_DEVICE_INFO

{

HI_CHAR aszServerSerialNumber[40 + 1];		//Equipment serial number

HI_CHAR aszServerSoftVersion[64 + 1];		//Software version

HI_CHAR aszServerName[40 + 1];			//Server name

HI_CHAR aszServerModel[40 + 1];			//Model

HI_CHAR aszStartDate[40 + 1];				//System startup date and time

HI_S32 s32ConnectState;					//Network connection status

}HI_DEVICE_INFO, *PHI_DEVICE_INFO;

Example:

HI_DEVICE_INFO sDeviceInfo;

ZK_NET_DEV_GetConfig (	lHandle,

ZK_NET_DEV_CMD_DEVICE_INFO,

&sDeviceInfo,

sizeof(HI_DEVICE_INFO));

- ZK_NET_DEV_CMD_PRODUCTID

Product IDs are represented by character strings.

Example:

HI_CHAR sID[64] = {0};

ZK_NET_DEV_GetConfig (lHandle, ZK_NET_DEV_CMD_PRODUCTID, sID, sizeof(sID));

- HI_ NET_DEV_CMD_USERNUM

It is used to obtain the number of users through int variables.

Example:

int nNum = 0;

ZK_NET_DEV_GetConfig (lHandle, ZK_NET_DEV_CMD_USERNUM, &nNum, sizeof(int));

- ZK_NET_DEV_CMD_SERVER_TIME

It is used to obtain the camera time.

typedef struct hiSERVERTIME_INFO_S

{

HI_CHAR sTime[32];			//Camera time, sample format: 20110311091208

} HI_S_SERVERTIME;

sTime indicates the camera time. Sample format: 20110311091208, that is 2011-3-11 09:12:08.

Example:

HI_S_SERVERTIME sServerTime;

ZK_NET_DEV_GetConfig (lHandle, ZK_NET_DEV_CMD_SERVER_TIME, & sServerTime, sizeof(HI_S_SERVERTIME));

## PTZ Control

You can determine whether a camera supports PTZ attributes based on the Z field of sProduct of ZK_NET_DEV_GET_PRODUCT_VENDOR. For details, see Appendix II "Vendor Code and Equipment Type Definitions."

### ZK_NET_DEV_PTZ_Ctrl_Standard

This function is used for PTZ control operations. Equipment containing the Z0 field does not support PTZ control.

HI_S32  ZK_NET_DEV_PTZ_Ctrl_Standard (

HI_U32 		u32Handle

HI_U32 		u32Command,

HI_U32 		u32Speed

);

Parameters

u32Handle

[IN] Operation handle

u32Command

[IN] PTZ control command

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_CTRL_PTZ_STOP | 0x3000 | Stopping PTZ |
| ZK_NET_DEV_CTRL_PTZ_UP | 0x3001 | Turning upward |
| ZK_NET_DEV_CTRL_PTZ_DOWN | 0x3002 | Turning downward |
| ZK_NET_DEV_CTRL_PTZ_LEFT | 0x3003 | Turning left |
| ZK_NET_DEV_CTRL_PTZ_RIGHT | 0x3004 | Turning right |
| ZK_NET_DEV_CTRL_PTZ_ZOOMIN | 0x3005 | Zooming in (the zoom ratio becomes larger) |
| ZK_NET_DEV_CTRL_PTZ_ZOOMOUT | 0x3006 | Zooming out (the zoom ratio becomes smaller) |
| ZK_NET_DEV_CTRL_PTZ_FOCUSIN | 0x3007 | Focusing in |
| ZK_NET_DEV_CTRL_PTZ_FOCUSOUT | 0x3008 | Focusing out |
| ZK_NET_DEV_CTRL_PTZ_APERTUREIN | 0x3009 | Aperture increase |
| ZK_NET_DEV_CTRL_PTZ_APERTUREOUT | 0x3010 | Aperture decrease |

u32Speed

[IN] Speed

#define ZK_NET_DEV_CTRL_PTZ_SPEED_MAX	0x3F //Maximum speed

#define ZK_NET_DEV_CTRL_PTZ_SPEED_MIN	0x00 //Minimum speed

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

The Z field of vendor IDs is used to determine whether an attribute is supported. The Z field is in sProduct of HI_S_ProductVendor.

### ZK_NET_DEV_PTZ_Ctrl_StandardEx

This function is used for PTZ control extension in single-step execution.

HI_S32  ZK_NET_DEV_PTZ_Ctrl_StandardEx (

HI_U32 		u32Handle

HI_U32 		u32Command,

);

Parameters

u32Handle

[IN] Operation handle

u32Command

[IN] PTZ control command

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_CTRL_PTZ_STOP | 0x3000 | Stopping PTZ |
| ZK_NET_DEV_CTRL_PTZ_UP | 0x3001 | Turning upward |
| ZK_NET_DEV_CTRL_PTZ_DOWN | 0x3002 | Turning downward |
| ZK_NET_DEV_CTRL_PTZ_LEFT | 0x3003 | Turning left |
| ZK_NET_DEV_CTRL_PTZ_RIGHT | 0x3004 | Turning right |
| ZK_NET_DEV_CTRL_PTZ_ZOOMIN | 0x3005 | Zooming in (the zoom ratio becomes larger) |
| ZK_NET_DEV_CTRL_PTZ_ZOOMOUT | 0x3006 | Zooming out (the zoom ratio becomes smaller) |
| ZK_NET_DEV_CTRL_PTZ_FOCUSIN | 0x3007 | Focusing in |
| ZK_NET_DEV_CTRL_PTZ_FOCUSOUT | 0x3008 | Focusing out |
| ZK_NET_DEV_CTRL_PTZ_APERTUREIN | 0x3009 | Aperture increase |
| ZK_NET_DEV_CTRL_PTZ_APERTUREOUT | 0x3010 | Aperture decrease |

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

PTZ control extension is used in single-step execution and motion.

### ZK_NET_DEV_PTZ_Ctrl_Preset

This function is used for PTZ preset position operations.

HI_S32  ZK_NET_DEV_PTZ_Ctrl_Preset (

HI_U32 		u32Handle

HI_U32 		u32Command,

HI_U32 		u32Preset

);

Parameters

u32Handle

[IN] Operation handle

u32Command

[IN] PTZ preset position control command

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_CTRL_PTZ_GOTO_PRESET | 0x3015 | Moving to a preset position |
| ZK_NET_DEV_CTRL_PTZ_SET_PRESET | 0x3016 | Configuring a preset position |
| ZK_NET_DEV_CTRL_PTZ_CLE_PRESET | 0x3017 | Clearing a preset position |

u32Preset

[IN] Preset position

#define ZK_NET_DEV_CTRL_PTZ_PRESET_MAX	255

#define ZK_NET_DEV_CTRL_PTZ_PRESET_MIN	0

Remarks

The Z field of vendor IDs is used to determine whether an attribute is supported. The Z field is in sProduct of HI_S_ProductVendor.

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_PTZ_Ctrl_Extend

This function is used for PTZ control extension.

HI_S32  ZK_NET_DEV_PTZ_Ctrl_Extend (

HI_U32 		u32Handle

HI_U32 		u32Command,

);

Parameters

u32Handle

[IN] Operation handle

u32Command

[IN] PTZ control command

| Macro Definition | Value | Meaning |
| --- | --- | --- |
| ZK_NET_DEV_CTRL_PTZ_LIGHT_ON | 0x3021 | Lighting on |
| ZK_NET_DEV_CTRL_PTZ_LIGHT_OFF | 0x3022 | Lighting off |
| ZK_NET_DEV_CTRL_PTZ_WIPER_ON | 0x3023 | Windscreen wiper on |
| ZK_NET_DEV_CTRL_PTZ_WIPER_OFF | 0x3024 | Windscreen wiper off |
| ZK_NET_DEV_CTRL_PTZ_AUTO_ON | 0x3025 | Automatically on |
| ZK_NET_DEV_CTRL_PTZ_AUTO_OFF | 0x3026 | Automatically off |
| HI_ NET_DEV_CTRL_PTZ_HOME | 0x3027 | Back to the central position |
| HI_ NET_DEV_CTRL_PTZ_CRUISE_V | 0x3028 | Vertical cruise |
| HI_ NET_DEV_CTRL_PTZ_CRUISE_H | 0x3029 | Horizontal cruise |

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

The Z field of vendor IDs is used to determine whether an attribute is supported. The Z field is in sProduct of HI_S_ProductVendor.

### ZK_NET_DEV_PTZ_Fully_Trans

This function is used for transparent PTZ operations.

HI_S32  ZK_NET_DEV_PTZ_Fully_Trans (

HI_U32 		u32Handle

HI_CHAR* 	psBuf,

HI_U32 		u32BufLen

);

Parameters

u32Handle

[IN] Operation handle

psBuf

[IN] Pointer of the buffer storing PTZ control codes

u32BufLen

[IN] PTZ control code length

#define ZK_NET_DEV_CTRL_PTZ_FT_BUF_LEN	64

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

## Voice Intercom

### ZK_NET_DEV_StartVoice

This function is used for transparent PTZ operations.

HI_S32  ZK_NET_DEV_StartpVoice (

HI_U32 		u32Handle,

HI_U32 		u32AudioType

);

Parameters

u32Handle

[IN] Operation handle

u32AudioType

[IN] Audio type (1–G.711, 4–G.726, and 5–AMR). Currently, only G.711 and G.726 are supported.

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

G.711 audio attributes:

struAAttr.eAEncode 		= PLAYERSDK_AUDIO_CODEC_FORMAT_G711A;

struAAttr.lBitRate			= 64000;

struAAttr.lSamplesPerSec	= 8000;

struAAttr.lBitsPerSample	= 8;

struAAttr.lBlockAlign		= 1;

struAAttr.lChannels		= 1;

struAAttr.length			= 0;

struAAttr.lFrameFlag		= 0;

struAAttr.pReserved		= NULL;

G.726 audio attributes:

struAAttr.eAEncode 		= PLAYERSDK_AUDIO_CODEC_FORMAT_G726;

struAAttr.lBitRate			= 16000;

struAAttr.lSamplesPerSec	= 8000;

struAAttr.lBitsPerSample	= 2;

struAAttr.lBlockAlign		= 1;

struAAttr.lChannels		= 1;

struAAttr.length			= 0;

struAAttr.lFrameFlag		= 0;

struAAttr.pReserved		= NULL;

### ZK_NET_DEV_StopVoice

This function is used for transparent PTZ operations.

HI_S32  ZK_NET_DEV_StopVoice (

HI_U32 		u32Handle

);

Parameters

u32Handle

[IN] Operation handle

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_SendVoiceData

This function is used for transparent PTZ operations.

HI_S32  ZK_NET_DEV_SendVoiceData (

HI_U32 		u32Handle

HI_CHAR	*psBuf,

HI_U32 		u32BufLen,

HI_U64 u64Pts

);

Parameters

u32Handle

[IN] Operation handle

psBuf

[IN] Audio data

u32BufLen

[IN] Data length

u64Pts

[IN] Timestamp

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

The format of the audio signal sent to a camera must be consistent with the audio type supported by the camera. For example, if the camera uses G.711, the audio signal sent to the camera must be G.711-compressed mono data of 8 kHz and 64 bits. If the camera uses G.726, the audio signal sent to the camera must be G.726-compressed mono data of 8 kHz and 16 bits.

## Video Recording

The composite streams obtained by callback functions can be directly recorded to files, which can be played by players directly. The composite video record format can be defined by users. The data obtained through network libraries can be saved as files. The specific format is as follows:

Example:

FILE  *pFile = NULL;

HI_S32 	OnStreamCallback(			//Real-time data callback by network libraries

HI_U32 	u32Handle, 		//Handle

HI_U32 	u32DataType,   	//Data type, which can be video data, audio data, or composite data of audios and videos

HI_U8*  pu8Buffer,     	//Data containing frame headers

HI_U32 	u32Length,      	//Data length

HI_VOID* pUserData)   	//User data

{

if (u32DataType == ZK_NET_DEV_SYS_DATA)

{

if (pFile != NULL)

{

fclose(pFile);

pFile = NULL;

}

pFile = fopen("d://video1.hx", "ab+");

if (NULL == pFile)

return;

fwrite(pu8Buffer, 1, u32Length, pFile);

}

else if (u32DataType == ZK_NET_DEV_AV_DATA)

{

if (NULL != pFile)

fwrite(pu8Buffer, 1, u32Length, pFile);

}

}

When u32DataType is ZK_NET_DEV_SYS_DATA, the pu8Buffer data structure is HI_S_SysHeader. When u32DataType is ZK_NET_DEV_AV_DATA, data contains frame header information HI_S_AVFrame and data blocks. The data block length is consistent with u32AVFrameLen of HI_S_AVFrame. The file structure of saved data is as follows:

HI_S_SysHeader

HI_S_AVFrame

Data block

HI_S_AVFrame

Data block

...................

HI_S_AVFrame

Data block

HI_S_AVFrame

Data block

Note: In the preceding example, video recording starts when data exists. To implement recording at a specified time, save the HI_S_SysHeader structure. When recording starts, save HI_S_SysHeader in front of a file and then store real-time streaming data.

For details about callback data, see the usage of ZK_NET_DEV_SetStreamCallBack.

Video records in user-defined formats can be played directly after being opened in play libraries. For details about recording callback, see the demo of play libraries.

### ZK_NET_DEV_StartRecord

This function is used for AVI recording.

HI_S32  ZK_NET_DEV_StartRecord (

HI_U32 		u32Handle,

HI_CHAR* 	psPath,

HI_U32 		u32Type,

HI_U32 		u32Flag

);

Parameters

u32Handle

[IN] Operation handle

psPath

[IN] Path + file name

u32Type

[IN] Record type

#define ZK_NET_DEV_VIDEO_AVI 0  	//AVI

#define ZK_NET_DEV_VIDEO_ASF 1		//ASF

#define ZK_NET_DEV_VIDEO_264 2		//User-defined format

u32Flag

[IN] Reserved field, which can be set to 0

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_StopRecord

This function is used to stop AVI recording.

HI_S32  ZK_NET_DEV_StopRecord (

HI_U32 		u32Handle

);

Parameters

u32Handle

[IN] Operation handle

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### ZK_NET_DEV_GetRecordState

This function is used to obtain the recording status.

HI_S32  ZK_NET_DEV_GetRecordState (

HI_U32 		u32Handle

);

Parameters

u32Handle

[IN] Operation handle

Return Values

HI_SUCCESS indicates that recording is in progress and HI_ FAILURE indicates no recording.

## Snapshot

### ZK_NET_DEV_SnapJpeg

This function is used for snapshot.

HI_S32 ZK_NET_DEV_SnapJpeg(

HI_U32 u32Handle,

HI_U8* pu8Data,

HI_S32 s32BufLen,

HI_S32 *pSize

);

Parameters

u32Handle

[IN] Operation handle

pu8Data

[IN] Memory address for picture saving

s32BufLen

[IN] Image resolution (width x height)

pSize

[IN] Valid length of pu8Data

Return Values

HI_SUCCESS is returned for successful snapshot and HI_ FAILURE for failed snapshot.

### ZK_NET_DEV_SnapJpegEx

This function is used for snapshot.

HI_S32 ZK_NET_DEV_SnapJpegEx(

HI_U32 u32Handle,

char *pFilePath,

int xRes,

int yRes,

int align );

Parameters

u32Handle

[IN] Operation handle

pFilePath

[IN] File path + file name

xRes

[IN] Snapshot resolution in the x direction

yRes

[IN] Snapshot resolution in the y direction

Align

[IN] 1: to the left (up); 2: to the middle; 3: to the right (down)

Return Values

HI_SUCCESS is returned for successful snapshot and HI_ FAILURE for failed snapshot.

# Audio Coding Description

The demo provides audio coding, audio acquisition by PCs, audio play, and voice intercom instances. For details, see the VC_demo provided by network libraries.

Encoding: Two different encoding formats are provided: G.711 and G.726.

Related classes: CHI_AENC_ENC and CHI_AI_MM

Decoding: Two decoding formats are provided: G.711 and G.726.

Related classes: CHI_ADEC_DEC and CHI_AO_MM

## Audio Collection Format Configuration

The audio attributes of audio adapters can be defined as follows:

m_waveformt.wFormatTag       =  WAVE_FORMAT_PCM;

m_waveformt.nChannels        =  1;

m_waveformt.nSamplesPerSec   =  8000;

m_waveformt.wBitsPerSample   =  16;

m_waveformt.cbSize           =  0;

m_waveformt.nBlockAlign      =  2;

m_waveformt.nAvgBytesPerSec  = 16000;

The configured audio format is PCM, in which a single channel is used, the collection frequency is 8000 Hz, and the size of data collected every second is 16000 bytes. The following relationship exists:

nBlockAlign       		=  nChannels * wBitsPerSample / 8 ;

nAvgBytesPerSec    	=  nSamplesPerSec * nBlockAlign ;

Note: Currently, audio signals are collected in units of 320 bytes because the G.711 and G.726 do not support 640-byte encoding.

## Audio Collection Process

Start the audio recording equipment: waveInOpen

Prepare buffers for the recording equipment: waveInPrepareHeader

Add buffers to the input equipment: waveInAddBuffer

Start recording: waveInStart

Clear buffers: waveInUnprepareHeader

Stop audio recording: waveInReset

Stop the audio recording equipment: waveInClose

## Audio Play Process

Start the output equipment: waveOutOpen

Prepare buffers for the output equipment: waveOutPrepareHeader

Import data to the buffer of the output equipment: waveOutWrite

Clear the output buffer: waveOutUnprepareHeader

Stop output: waveOutReset

Stop the output equipment: waveOutClose

## Audio Encoding

Configure the audio encoding library hisi_voice_engine.lib, which supports only G.711A and G.726. Configure the encoding format and initialize encoding libraries based on the structure PLAYER_ATTR_AUDIO_S used in audio configuration.

HRESULT CHI_AENC_ENC::HI_AENC_Init(PLAYER_ATTR_AUDIO_S *pAudioAttr)

{

HI_S32 s32Rel;

HI_U16 u16CodeType;

if (NULL == pAudioAttr)

{

return HI_ERR_NULL_POINTER;

}

if (PLAYER_AUDIO_CODEC_FORMAT_G711A == pAudioAttr->eAEncode)

{

m_pAEncHandle = &g711_dec_state;

u16CodeType = G711_A;

}

else if (PLAYER_AUDIO_CODEC_FORMAT_G711U == pAudioAttr->eAEncode)

{

m_pAEncHandle = &g711_dec_state;

u16CodeType = G711_U;

}

else if (PLAYER_AUDIO_CODEC_FORMAT_G726 == pAudioAttr->eAEncode)

{

m_pAEncHandle = &g726_dec_state;

if (16000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_16KBPS;

}

else if (24000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_24KBPS;

}

else if (32000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_32KBPS;

}

else if (40000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_40KBPS;

}

else

{

return HI_ERR_INVALID_ARGUMENTS;

}

}

else

{

return HI_ERR_INVALID_ARGUMENTS;

}

/*Reinitialize encoding*/

s32Rel = HI_VOICE_EncReset(m_pAEncHandle, u16CodeType);

return (s32Rel == HI_SUCCESS) ? HI_SUCCESS : HI_FAILURE;

}

In the preceding code:

#define G711_A         0x01  /* 64kbps G.711 A, see RFC3551.txt  4.5.14 PCMA */

#define G711_U         0x02  /* 64kbps G.711 U, see RFC3551.txt  4.5.14 PCMU */

#define MEDIA_G726_16KBPS             0x24  /* G726 16kbps for ASF ... */

#define MEDIA_G726_24KBPS             0x25  /* G726 24kbps for ASF ... */

#define MEDIA_G726_32KBPS             0x26  /* G726 32kbps for ASF ... */

#define MEDIA_G726_40KBPS             0x27  /* G726 40kbps for ASF ... */

Audio encoding function:

HI_VOICE_API HI_RESULT  HI_VOICE_EncodeFrame (

HI_VOID 	*pEncState, // HI_VOICE_EncReset handle

HI_S16 		*pInputBuf,	//Data that is not encoded

HI_S16 		*pOutputBuf,	//Encoded data

HI_S16 		len			//Size of the data that is not encoded

);

Size of the encoded data:

*pOutLen = (pOutBuf[2])*2 + 4; //Code stream data added with HISI frame structure headers

## Audio Decoding

Configure the audio decoding library hisi_voice_engine.lib, which supports only G.711A and G.726. Configure the decoding format and initialize decoding based on the audio configuration structure PLAYER_ATTR_AUDIO_S.

HRESULT CHI_ADEC_DEC::HI_ADEC_Init(PLAYER_ATTR_AUDIO_S *pAudioAttr,

PLAYER_ATTR_AUDIO_S *pOutPutAttr)

{

if (NULL == pAudioAttr || NULL == pOutPutAttr)

{

return HI_ERR_NULL_POINTER;

}

HI_U16 u16CodeType;

if (PLAYER_AUDIO_CODEC_FORMAT_G711A == pAudioAttr->eAEncode)

{

m_pDecCodeState = &g711_dec_state;

u16CodeType = G711_A;

}

else if (PLAYER_AUDIO_CODEC_FORMAT_G711U == pAudioAttr->eAEncode)

{

m_pDecCodeState = &g711_dec_state;

u16CodeType = G711_U;

}

else if (PLAYER_AUDIO_CODEC_FORMAT_G726 == pAudioAttr->eAEncode)

{

m_pDecCodeState = &g726_dec_state;

if (16000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_16KBPS;

}

else if (24000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_24KBPS;

}

else if (32000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_32KBPS;

}

else if (40000 == pAudioAttr->lBitRate)

{

u16CodeType = MEDIA_G726_40KBPS;

}

else

{

return HI_ERR_INVALID_ARGUMENTS;

}

}

else

{

return HI_ERR_INVALID_ARGUMENTS;

}

/*Reinitialize decoding*/

HI_S32 s32Rel = HI_VOICE_DecReset(m_pDecCodeState, u16CodeType);

if (HI_SUCCESS != s32Rel)

{

return HI_FAILURE;

}

return HI_SUCCESS;

}

In the preceding code:

#define G711_A         0x01  /* 64kbps G.711 A, see RFC3551.txt  4.5.14 PCMA */

#define G711_U         0x02  /* 64kbps G.711 U, see RFC3551.txt  4.5.14 PCMU */

#define MEDIA_G726_16KBPS             0x24  /* G726 16kbps for ASF ... */

#define MEDIA_G726_24KBPS             0x25  /* G726 24kbps for ASF ... */

#define MEDIA_G726_32KBPS             0x26  /* G726 32kbps for ASF ... */

#define MEDIA_G726_40KBPS             0x27  /* G726 40kbps for ASF ... */

Audio decoding function:

HI_VOICE_DecodeFrame(

HI_VOID 	*pDecState, // HI_VOICE_DecReset handle

HI_S16 		*pInputBuf,	//Data that is not decoded

HI_S16 		*pOutputBuf,	//Decoded data

HI_S16 		*pLen		//Size of decoded data

);

# Appendix

## Folder List

Lib: stores library files ZKNetLib.lib, ZKPlayer.lib, and ZKSearchLib.lib.

Include: stores header files.

VC_demo: stores the mfc demo.

CamWizardDemo: stores the equipment search tool demo.

Bin: stores executable files.

## Vendor Code and Equipment Type Definitions

- Vendor code

A vendor code is used to identify a vendor and contains 32 ASCII-coded bytes. It can be modified by special tools and is read-only for users.

- Equipment type

Equipment types are used to identify equipment. Equipment functions vary with different equipment. An equipment type contains 32 ASCII-coded bytes. It can be modified by special tools and is read-only for users.

Each field contains two bytes. The first byte indicates the general field type and the second byte indicates the field subtype.

| Field 1 | Field 2 | Field 3 | Field 4 | Field 5 | Field 6 | Field 7 | Reserved field |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chip | Format | Shot | PTZ type | Network type | Platform type | Language type |  |
| 'C' | 'F' | 'S' | 'Z' | 'N' | 'P' | 'L' |  |

(1) Chip field 'C'

Chip types include the following:

| '0' | Hi3510 |
| --- | --- |
| '1' | Hi3512 |

(2) Format field 'F'

Video input formats. The current values are as follows:

| '0' | PAL and NTS are supported. |
| --- | --- |
| '1' | PAL(704x576, 352x288, 176x144). The maximum frame rate is 25 frames per second (FPS). |
| '2' | NTSC(704x480, 352x240, 176x120). The maximum frame rate is 30 FPS. |

(3) Shot field 'S'

Sensor types include the following:

| '0' | OV7725 infrared control | Lightness, contrast, saturation, chromaticity, indoor, outdoor, infrared switch, vertical flip, and left and right mirroring Primary code streams: VGA, QVGA, and QQVGA Secondary code streams: QVGA and QQVGA |
| --- | --- | --- |
| '1' | CCDOSP | Lightness, contrast, saturation, and chromaticity Primary code streams: D1, CIF, and QCIF Secondary code streams: CIF and QCIF |
| '2' | CCD | Lightness, contrast, saturation, and chromaticity Primary code streams: D1, CIF, and QCIF Secondary code streams: CIF and QCIF |
| '3' | MT9D131 | Lightness, contrast (1–7), saturation, vertical flip, and left and right mirroring Primary code stream: 720 P (maximum frame rate: 30 FPS) Secondary code stream: QVGA |
| '4' | HDCCD | Primary code stream: 720 P (maximum frame rate: 30 FPS) Secondary code stream: QVGA |
| '5' | 630D | Lightness (0–6), contrast (0–8), and saturation (0–6) Primary code stream: 720 P (maximum frame rate: 30 FPS) Secondary code stream: Q720P |
| '6' | 630C | Lightness (0–4), contrast (0–4), and saturation (0–2) Primary code stream: 720 P (maximum frame rate: 30 FPS) Secondary code stream: Q720P |
| '7' | CMOS 720P | Lightness, contrast (1–7), saturation, vertical flip, and left and right mirroring Primary code streams: 720P (maximum frame rate: 30 FPS) and Q720P Secondary code streams: Q720P and QQ720P |
| '8' | 633 | Lightness (0–6), contrast (0–8), and saturation (0–6) Primary code streams: 720P (maximum frame rate: 30 FPS) and Q720P Secondary code streams: Q720P and QQ720P |
| '9' | CMOS 200M | Lightness, contrast (1–7), saturation, vertical flip, and left and right mirroring Primary code streams: 720P (maximum frame rate: 30 FPS) and Q720P Secondary code streams: Q720P and QQ720P UXGA (maximum frame rate: 15 FPS), VGA secondary code stream, and QVGA |

(4) PTZ field 'Z'

PTZ types include the following:

| '0' | Small dome | Up, down, left, right, vertical cruise, horizontal cruise, back to the central position, and switch to a preset position (a maximum of eight positions) Serial port configuration is unavailable. |
| --- | --- | --- |
| '1' | White dome | Up, down, left, right, and switch to a preset position (a maximum of eight positions) Serial port configuration is fixed. |
| '2' | Zoom dome | Up, down, left, right, zoom, and switch to a preset position (a maximum of eight positions) Serial port configuration is fixed. |
| '3' | Standard | Up, down, left, right, windscreen wiper, lighting, and switch to a preset position Serial ports can be configured |
| '4' | Small zoom dome | Up, down, left, right, vertical cruise, horizontal cruise, back to the central position, zoom in, and zoom out |

(5) Network field 'N'

Network types include the following:

| '0' | Wired networks are supported. |
| --- | --- |
| '1' | Wi-Fi is supported. |
| '2' | EVDO is supported. |
| '3' | TD is supported. |
| '4' | WCDMA is supported. |

(6) Platform field 'P'

Platform types include the following:

| '0' | Platforms are unavailable. |
| --- | --- |

(7) Language field 'L'

Language types include the following:

| '0' | Chinese |
| --- | --- |
| '1' | English |

(8) The reserved field is used for future expansion.