Search Library Usage Guide

Version: 3.0.0.1

Description of the Search Library SDK	2

1.1 Programming Guide	2

1.2 Data Structure	2

1.3 Interface Description	3

1.3.1 Initializing Equipment Search	3

1.3.2 Unintializing Equipment Search	4

1.3.3 Registering the Search Response Processing Function	4

1.3.4 Registering the Command Response Processing Function	5

1.3.5 Registering the Local IP Address for Receiving Search Responses	6

1.3.6 Sending Search Commands	6

1.3.7 Sending Configuration Commands	6

Appendix	8

I Folder List	8

II Vendor Code and Equipment Type Definitions	8

# Description of the Search Library SDK

Version: 3.0.0.1

## 1.1 Programming Guide

Start searching.

ZK_VSCP_DEVSCLI_Search()

Search for callback functions to obtain data.

PTR_VSCP_DEVS_Search_RPNProcFN

Set the parameters of a network camera.

ZK_VSCP_DEVSCLI_Cmd()

Uninitialize the search library.

ZK_VSCP_DEVSCLI_Deinit()

Register the search result callback function.

ZK_VSCP_DEVSCLI_RegistSearchProc()

Register the IP address of the local PC.

ZK_VSCP_DEVSCLI_Register_IP()

Initialize a search library.

ZK_VSCP_DEVSCLI_INIT()

## 1.2 Data Structure

Equipment stream information:

typedef struct {

HI_CHAR aszIP[HI_VSCP_IP_STRSIZE + 1];              /*IP address*/

HI_CHAR aszMASK[HI_VSCP_IP_STRSIZE + 1];            /*Subnet mask*/

HI_CHAR aszMAC[HI_VSCP_MAC_STRSIZE + 1];            /*MAC address*/

HI_CHAR aszIP[HI_VSCP_IP_STRSIZE + 1];              /*Gateway address*/

HI_S32    s32Dhcp;     /* DHCP. 1 indicates that DHCP is enabled and 0 indicates that DHCP is disabled.*/

HI_S32    s32DnsFlag; /*DNS configuration flag. 1 indicates that DNS configuration is automatic and 0 indicates that DNS configuration is manual.*/

HI_CHAR aszFdns[HI_VSCP_IP_STRSIZE + 1];    /*Preferred DNS */

HI_CHAR aszSdns[HI_VSCP_IP_STRSIZE + 1];   /*Backup DNS */

} HI_S_VSCP_NETINFO;

typedef struct {

HI_CHAR aszDevID[HI_VSCP_DEVID_STRSIZE + 1]; //Equipment ID, which contains 32 randomly generated characters

HI_CHAR aszDevMDL[HI_VSCP_DEVNAME_STRSIZE + 1]; //Equipment model

HI_CHAR aszSwVersion[HI_VSCP_SWVER_STRSIZE + 1]; //Software version

HI_CHAR aszDevName[HI_VSCP_DEVNAME_STRSIZE + 1]; //Equipment name

HI_CHAR aszHttpPort[HI_VSCP_IP_STRSIZE + 1]; //HTTP listening port

HI_S_VSCP_NETINFO struNetInfo;

} HI_S_VSCP_DEVINFO;

Information about the target equipment for sending commands:

typedef  struct{

HI_CHAR*  pszDevID; //Equipment ID, which is the unique identification of equipment. It is searchable.

HI_CHAR*  pszUserName; //User name

HI_CHAR*  pszPasswd; //Password

}  HI_S_VSCP_DEVSCLI_DevInfo;

## 1.3 Interface Description

### 1.3.1 Initializing Equipment Search

The code for initialization is as follows:

HI_S32  ZK_VSCP_DEVSCLI_INIT (

const HI_CHAR*  	pszListenIP,

HI_U16  			u16Port,

HI_U32  			u32TimeOut,

HI_VOID**  		ppvHandle

);

Parameters

pszListenIP

[IN] Multicast IP address for search response processing, which is permanently set to 239.255.255.250

u16Port

[IN] Multicast port number for search response processing, which is permanently set to 8002

u32TimeOut

[IN] Search timeout value, in the unit of seconds

ppvHandle

[IN] Handle for search object output

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### 1.3.2 Unintializing Equipment Search

The code for uninitialization is as follows:

HI_S32  ZK_VSCP_DEVSCLI_Deinit (

HI_VOID*  	pvHandle

);

Parameters

pvHandle

[IN] Handle for search object output

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### 1.3.3 Registering the Search Response Processing Function

The code for registering the search response processing function is as follows:

HI_S32  ZK_VSCP_DEVSCLI_Deinit (

HI_VOID*  			pvHandle,

PTR_VSCP_DEVS_Search_RPNProcFN  pfunSearchRProc,

HI_VOID*  			pvUserData

);

Parameters

pvHandle

[IN] Search object handle

pfunSearchRProc

[IN] Callback function for search response processing

pvUserData

[IN] User data, which is sent by the callback function for search response processing

Callback Function

typedef  HI_S32  (*PTR_VSCP_DEVS_Search_RPNProcFN) (

const  HI_VOID*  		pvHandle,

HI_CHAR*  				pszRNPCode,

HI_S_VSCP_DEVINFO*  	pstruDevInfo,

HI_VOID*  				pvUserData

}

Callback Function Parameters

pvHandle

Unused

pszRNPCode

Return value

pstruDevInfo

Equipment information

pvUserData

User data

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

Remarks

The channel and code stream attributes corresponding to the pstruDevInfo parameter are allocated with space by the SDK internally. Therefore, the upper-layer application must call the free function to release the space after use of the pstruDevInfo parameter. For implementation details, see the demo source code provided by the SDK.

### 1.3.4 Registering the Command Response Processing Function

The code for registering the command response processing function is as follows:

HI_S32  ZK_VSCP_DEVSCLI_RegistCmdProc (

HI_VOID*  			pvHandle,

PTR_VSCP_DEVS_Cmd_RPNProcFN  pfunCmdRProc,

HI_VOID*  			pvUserData

);

Parameters

pvHandle

[IN] Search object handle

pfunCmdRProc

[IN] Command response processing function

pvUserData

[IN] User data, which is sent by the callback function for command response processing

Callback Function

typedef  HI_S32  (*PTR_VSCP_DEVS_Cmd_RPNProcFN) (

const  HI_VOID*  		pvHandle,

HI_CHAR*  				pszRNPCode,

HI_S_VSCP_DEVSCLI_Cmd_ResponsInfo*  pstruResponseInfo,

HI_VOID*  				pvUserData

}

Callback Function Parameters

pvHandle

Unused

pszRNPCode

Return value. Configuration is successful when the return value contains 200; otherwise, configuration fails.

pstruResponseInfo

Unused

pvUserData

User data

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### 1.3.5 Registering the Local IP Address for Receiving Search Responses

The code for registering the local IP address for receiving search responses is as follows:

HI_S32  ZK_VSCP_DEVSCLI_Register_IP (

HI_CHAR  	aaszIP[][HI_VSCP_IP_STRSIZE+1],

HI_U32  	u32Num

);

Parameters

aaszIP

[OUT] Local IP address list

u32Num

[OUT] Number of local IP addresses

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### 1.3.6 Sending Search Commands

The code for sending search commands is as follows:

HI_S32  ZK_VSCP_DEVSCLI_Search (

HI_VOID*  	pvHandle

);

Parameters

pvHandle

[IN] Search object handle

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

### 1.3.7 Sending Configuration Commands

The code for sending configuration commands is as follows:

HI_S32  ZK_VSCP_DEVSCLI_Cmd (

HI_VOID*  							pvHandle,

const  HI_S_VSCP_DEVSCLI_DevInfo	*pstruDEV,

HI_S32 								s32Cmd,

const HI_VOID* 						pData

);

Parameters

pvHandle

[IN] Search object handle

PstruDEV

[IN] Equipment information:

s32Cmd

[IN] Configuration type

#define HI_VSCP_CMD_NET        0x01		//Basic network parameter

#define HI_VSCP_CMD_PORT       0x02		//Port number

pData

[IN] Parameter configuration

- HI_VSCP_CMD_NET: HI_S_VSCP_NETINFO structure

typedef struct {

HI_CHAR aszIP[HI_VSCP_IP_STRSIZE + 1];		//IP address

HI_CHAR aszMASK[HI_VSCP_IP_STRSIZE + 1];		//Subnet mask

HI_CHAR aszMAC[HI_VSCP_MAC_STRSIZE + 1];		//MAC address

HI_CHAR aszGTW[HI_VSCP_IP_STRSIZE + 1];		//Gateway address

HI_S32    s32Dhcp;     			//DHCP. 1 indicates that DHCP is enabled and 0 indicates that DHCP is disabled.*/

HI_S32    s32DnsFlag; 		//DNS configuration flag. 1 indicates that DNS configuration is automatic and 0 indicates that DNS configuration is manual.

HI_CHAR aszFdns[HI_VSCP_IP_STRSIZE + 1];		//Preferred DNS

HI_CHAR aszSdns[HI_VSCP_IP_STRSIZE + 1];		//Backup DNS

} HI_S_VSCP_NETINFO;

- HI_VSCP_CMD_PORT: char str[16]

Return Values

HI_SUCCESS is returned for a successful operation and HI_ FAILURE for a failed operation.

# Appendix

## I Folder List

Lib: stores library files ZKNetLib.lib, ZKPlayer.lib, and ZKSearchLib.lib.

Include: stores header files.

VC_demo: stores the mfc demo.

Bin: stores executable files.

## II Vendor Code and Equipment Type Definitions

- Vendor code

A vendor code is used to identify a vendor and contains 32 ASCII-coded bytes. It can be modified by special tools and is read-only for users.

- Equipment type

Equipment types are used to identify equipment. Equipment functions vary with different equipment. An equipment type contains 32 ASCII-coded bytes. It can be modified by special tools and is read-only for users.

Each field contains two bytes. The first byte indicates the general field type and the second byte indicates the field subtype.

| Field 1 | Field 2 | Field 3 | Field 4 | Field 5 | Field 6 | Field 7 | Reserved field |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chip | Format | Shot | Pan Tilt Zoom (PTZ) type | Network type | Platform type | Language type |  |
| 'C' | 'F' | 'S' | 'Z' | 'N' | 'P' | 'L' |  |

(1) Chip field 'C'

Chip types include the following:

| '0' | Hi3510 |
| --- | --- |
| '1' | Hi3512 |

(2) Format field 'F'

Video input formats. The current values are as follows:

| '0' | PAL and NTS are supported. |
| --- | --- |
| '1' | PAL(704x576, 352x288, 176x144). The maximum frame rate is 25 frames per second (FPS). |
| '2' | NTSC(704x480, 352x240, 176x120). The maximum frame rate is 30 FPS. |

(3) Shot field 'S'

Sensor types include the following:

| '0' | OV7725 infrared control | Lightness, contrast, saturation, chromaticity, indoor, outdoor, infrared switch, vertical flip, and left and right mirroring Primary code streams: VGA, QVGA, and QQVGA Secondary code streams: QVGA and QQVGA |
| --- | --- | --- |
| '1' | CCDOSP | Lightness, contrast, saturation, and chromaticity Primary code streams: D1, CIF, and QCIF Secondary code streams: CIF and QCIF |
| '2' | CCD | Lightness, contrast, saturation, and chromaticity Primary code streams: D1, CIF, and QCIF Secondary code streams: CIF and QCIF |
| '3' | MT9D131 | Lightness, contrast (1–7), saturation, vertical flip, and left and right mirroring Primary code stream: 720P (maximum frame rate: 30 FPS) Secondary code stream: QVGA |
| '4' | HDCCD | Primary code stream: 720P (maximum frame rate: 30 FPS) Secondary code stream: QVGA |
| '5' | 630D | Lightness (0–6), contrast (0–8), and saturation (0–6) Primary code stream: 720P (maximum frame rate: 30 FPS) Secondary code stream: Q720P |
| '6' | 630C | Lightness (0-4), contrast (0-4), and saturation (0-2) Primary code stream: 720P (maximum frame rate: 30 FPS) Secondary code stream: Q720P |
| '7' | CMOS 720P | Lightness, contrast (1–7), saturation, vertical flip, and left and right mirroring Primary code streams: 720P (maximum frame rate: 30 FPS) and Q720P Secondary code streams: Q720P and QQ720P |
| '8' | 633 | Lightness (0–6), contrast (0–8), and saturation (0–6) Primary code streams: 720P (maximum frame rate: 30 FPS) and Q720P Secondary code streams: Q720P and QQ720P |
| '9' | CMOS 200M | Lightness, contrast (1–7), saturation, vertical flip, and left and right mirroring Primary code streams: 720P (maximum frame rate: 30 FPS) and Q720P Secondary code streams: Q720P and QQ720P UXGA (maximum frame rate: 15 FPS), VGA secondary code stream, and QVGA |

(4) PTZ field 'Z'

PTZ types include the following:

| '0' | Small dome | Up, down, left, right, vertical cruise, horizontal cruise, back to the central position, and switch to a preset position (a maximum of eight positions) Serial port configuration is unavailable. |
| --- | --- | --- |
| '1' | White dome | Up, down, left, right, and switch to a preset position (a maximum of eight positions) Serial port configuration is fixed. |
| '2' | Zoom dome | Up, down, left, right, zoom, and switch to a preset position (a maximum of eight positions) Serial port configuration is fixed. |
| '3' | Standard | Up, down, left, right, windscreen wiper, lighting, and switch to a preset position Serial ports can be configured |
| '4' | Small zoom dome | Up, down, left, right, vertical cruise, horizontal cruise, back to the central position, zoom in, and zoom out |

(5) Network field 'N'

Network types include the following:

| '0' | Wired networks are supported. |
| --- | --- |
| '1' | Wi-Fi is supported. |
| '2' | EVDO is supported. |
| '3' | TD is supported. |
| '4' | WCDMA is supported. |

(6) Platform field 'P'

Platform types include the following:

| '0' | Platforms are unavailable. |
| --- | --- |

(7) Language field 'L'

Language types include the following:

| '0' | Chinese |
| --- | --- |
| '1' | English |

(8) The reserved field is used for future expansion.